# COMPUTER ORGANISATION AND ARCHITECTURE
LAB

FAROOQ AHMAD 								EXPERIMENT # 10
17L4407									LAB REPORT # 10
BEE-6A1

EXECUTION TIME

## INTRODUCTION

In this lab session, we were introduced to the Execution Time. Instruction cycle is the time period during which one instruction is fetched from memory and executed when a computer is given an instruction in machine language. In a basic computer, each instruction cycle consists of the following phases: Fetch instruction from memory, decode the instruction, read the effective address from memory and execute the instruction.  Cycles per Instruction, also known as clock cycles per instruction, or clocks per instruction (CPI) is the number of clock cycles that happen when an instruction is being executed by a computer with a given clock frequency. It can be taken as an average over a typical sequence of instructions. In any case, it is better used when applied to RISC processors. In these processors, theoretically, most instructions should be able to complete in one computer cycle.

The execution time of a program consists of many factors, most of which are ignored (such as waits for I/O, instruction fetch times, pipeline delays, etc.) Execution Time (processor-related) = IC x CPI x T where,

IC = instruction count

CPI = average number of system clock periods to execute an instruction T = clock period

## OBJECTIVE

The course learning objective of this lab session was understand and learn

- How to calculate the execution time for a piece of code.

- How to interpret the different execution times of various instructions.

- How to write efficient codes.

- How to evaluate a particular program with respect to its performance measure

## PROCEDURE

Question 1

Code A

Type 		no of instructions 	CPI	 total cycles

Control 		9 		2 		18

Data			9		3		27

ALSU 			2		4		18

Total execution time = 18+27+18 = 63

Code B

Type 		no of instructions 	CPI 	total cycles

Control 		20 		2 		40

Data 			12 		3 		36

ALSU 			20 		4 		80

Total execution time = 40+36+80 = 156

Question 2

[org 0x0100]

mov al,[M]

mov bl,[N]

;M divided by N

;al mein aye ga quotient

;ah mein aye ga remainder

again:

div bl

cmp ah,0

je exit

mov ax,bx

mov bl,ah

jmp again

exit:

mov [GCD],bl

jmp end

M:db 12

N:db 8

GCD: db 00h

end:

mov ax, 0x4c00 ; terminate program

int 0x21

## ISSUES

There were no issues.

## CONCLUSION

The instruction cycle is the time period during which one instruction is fetched from memory and executed when a computer is given an instruction in machine language. There are typically five stages of an instruction cycle that the CPU carries out: Fetch the instruction, Decode the instruction, Read the effective address from memory if the instruction has an indirect address, Execute the instruction, Store in memory any results generated by the operation, or send the results to an output device. Execution Time (processor-related) = IC x CPI x T.

## APPLICATIONS

We can easily calculate the efficiency of our program. The lower the execution time, the greater the efficiency.