# COMPUTER ORGANISATION AND ARCHITECTURE
LAB

FAROOQ AHMAD 								EXPERIMENT # 13
17L4407									LAB REPORT # 13
BEE-6A1

STRING INSTRUCTIONS

## INTRODUCTION

In this lab session, we were introduced to the string instructions. String is s series of data byte or word available in memory at consecutive locations. It is either referred as byte string or word string. Their memory is always allocated in a sequential order. Instructions used to manipulate strings are called string manipulation instructions.
The 8086 supports a set of more powerful instructions for string manipulations for referring to a string, two parameters are required. I. Starting and End Address of the String. II. Length of the String. The length of the string is usually stored as count in the CX register. The incrementing or decrementing of the pointer, in string instructions, depends upon the Direction Flag (DF) Status. If it is a Byte string operation, the index registers are updated by one. On the other hand, if it is a word string operation, the index registers are updated by two.

## OBJECTIVE

The course learning objective of this lab session was

- To be able to use string instructions

- To be able to use index registers SI and DI for string operations.

## PROCEDURE

Question 1

[ORG 0x0100]

mov cx,7

push ds

push es

cld;

mov ax,str1

mov si,ax

mov ax,str2

mov di,ax

repe cmpsb

MOV AX, 0x4C00 ; TERMINATE PROGRAM

INT 0x21

str1:

db 'My name is Khan and I am not A TERRORIST',0

str2:

db 'My larger string',0

Question 2

[ORG 0x0100]

mov cx,17

push ds

push es

cld

mov ax,str1

mov si,ax

mov ax,str2

mov di,ax

repe cmpsb

mov cx,17

jc str2large

;str1larger

mov ax,str1

mov si,ax

mov ax,larger

mov di,ax

repe movsb

mov ax,str2

mov si,ax

mov ax,smaller

mov di,ax

repe movsb

jmp end_code

str2large:

mov ax,str2

mov si,ax

mov ax,larger

mov di,ax

rep movsb

mov ax,str1

mov si,ax

mov ax,smaller

mov di,ax

rep movsb

end_code:

MOV AX, 0x4C00 ; TERMINATE PROGRAM

INT 0x21

str1:

db 'My name is Khan and I am not A TERRORIST',0

str2:

db 'My larger string',0

larger:

db 0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0

smaller:

db 0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0

## ISSUES

There were no issues.

## CONCLUSION

String instructions work on a block of data. A block has a start and an end. The instructions can work from the start towards the end and from the end towards the start. In fact they can work in both directions, and they must be allowed to work in both directions otherwise certain operations with overlapping blocks become impossible.

## APPLICATIONS

Instruction pipelining is a technique used in the design of modern microprocessors, microcontrollers and CPUs to increase their instruction throughput. Pipelining is used in almost all computer architectures, to speed up the processes and improve efficiency. The five instructions are STOS, LODS, CMPS, SCAS, and MOVS called store string, load string, compare string, scan string, and move string respectively.