# COMPUTER ORGANISATION AND ARCHITECTURE
LAB

FAROOQ AHMAD 								EXPERIMENT # 3
17L4407									LAB REPORT # 3
BEE-6A1

MEMORY SYSTEM

## INTRODUCTION

In this lab session, we were introduced to the memory system. Segmentation is the process in which the main memory of the computer is divided into different segments and each segment has its own base address. It is basically used to enhance the speed of execution of the computer system, so that processor is able to fetch and execute the data from the memory easily and fast.

Big-endian and little-endian are terms that describe the order in which a sequence of bytes is stored in computer memory. Big-endian is an order in which the "big end" (most significant value in the sequence) is stored first (at the lowest storage address). Little-endian is an order in which the "little end" (least significant value in the sequence) is stored first.

The Bus Interface Unit (BIU) contains four 16 bit special purpose registers (mentioned below) called as Segment Registers.

- Code segment register (CS): is used for addressing memory location in the code segment of the memory, where the executable program is stored.

- Data segment register (DS): points to the data segment of the memory where the data is stored.

- Extra Segment Register (ES): also refers to a segment in the memory which is another data segment in the memory.

- Stack Segment Register (SS): is used for addressing stack segment of the memory. The stack segment is that segment of memory which is used to store stack data.

## OBJECTIVE

The course learning objective of this lab session was to be able to do 32-bit computations using 16-bit registers and learn the x86 memory organization and segmentation..

## PROCEDURE

The number of address lines in 8086 is 20, 8086 BIU will send 20bit address, to access one of the 1MB memory locations. The four segment registers contain the upper 16 bits of the starting addresses of the four memory segments of 64 KB each with which the 8086 is working at that instant of time. A segment is a logical unit of memory that may be up to 64 kilobytes long.

Given a logic address with following syntax we can calculate physical address as specified below:

Logical Address = Segment: Offset (The 16-bit segment, 16-bit offset)

Physical Address (20-bit address) = Segment * 10h + Offset

In this lab we stored 0xABDADDBA in the two storage forms.

Little Endian

| Address | Contents |
| --- | --- |
| 1996 | BA |
| 1997 | DD |
| 1998 | DA |
| 1999 | AB |

Big Endian

| Address | Contents |
| --- | --- |
| 1996 | AB |
| 1997 | DA |
| 1998 | DD |
| 1999 | BA |

Next two physical memory addresses were added:

- 1DDD:0436:

1DDD0 + 00436=0x1E206

- 1234:7920

12340 + 07920 =19C60

- 74F0:2123

74F00 + 02123=0x77023

- 0000:6727:

00000 + 06727=0x6727

- FFFF:4336:

FFFF0 + 04336=0x104326

; A PROGRAM TO ADD TWO NUMBERS AND SUBTRACT A THIRD ONE FROM THE SUM

; USING REGISTERS

[ORG 0x0100]

MOV AX,0

MOV BX,0

HERE:

INC BX

ADD AX,20

CMP BX,20

JLE HERE

MOV AX, 0x4C00 ; TERMINATE PROGRAM

INT 0x21

Q3

; A PROGRAM TO ADD TWO NUMBERS AND SUBTRACT A THIRD ONE FROM THE SUM

; USING REGISTERS

[ORG 0x0100]

MOV AX,0x1234

MOV [DS:0x0025],AX

MOV AX,[DS:0x0025]

MOV AX,0

MOV AL,[DS:0x0025]

MOV AX,[DS:0x0010]

MOV [DS:0x002F],AX

MOV AX, 0x4C00 ; TERMINATE PROGRAM

INT 0x21

Q4

; A PROGRAM TO ADD TWO NUMBERS AND SUBTRACT A THIRD ONE FROM THE SUM

; USING REGISTERS

[ORG 0x0100]

MOV AX,0x0FFF

MOV BX,0

HERE:

ADD AX,1

ADC BX,0

JMP HERE

MOV AX, 0x4C00 ; TERMINATE PROGRAM

INT 0x21

; A PROGRAM TO ADD TWO NUMBERS AND SUBTRACT A THIRD ONE FROM THE SUM

; USING REGISTERS

[ORG 0x0100]

MOV AX,10

MOV BX,110

HERE:

SUB AX,1

SBB BX,0

JMP HERE

MOV AX, 0x4C00 ; TERMINATE PROGRAM

INT 0x21

## ISSUES

There were no issues.

## CONCLUSION

The memory in 8086 is segmented and there are two ways to store things in memory, little endian and big endian. Big endian stores higher byte in lower memory address and little-endian stores higher byte in higher memory address. For segmented memory, 20-bit memory address is calculated by first adding four zero bits at the end of segment and then adding it to the offset. Also, we can now apply the memory concepts to perform 32-bit manipulations.

## APPLICATIONS

The DOSBox emulator is an application that runs as a stand-alone program that can be used to run thousands of old games and programs using the original files, floppy disks or CD-ROMs.
 The registers on the 8086 are used to perform various arithmetic and logical operations such as 32-bit addition and subtraction. These concepts can be used to store and move data greater than 16 bits in different addresses. Also, 32-bit manipulations are used in different devices such as calculator.

## POSTLAB

[ORG 0x100]

mov ax,0x0000

mov bx,0xFFFF

mov cx,0x1234

mov dx,0x5678

adc bx,dx

jc carry

jmp end

carry:

adc ax,cx

end:

MOV AX, 0x4C00 ; TERMINATE PROGRAM

INT 0x21