# COMPUTER ORGANISATION AND ARCHITECTURE
LAB

FAROOQ AHMAD 								EXPERIMENT # 7
17L4407									LAB REPORT # 7
BEE-6A1

BRANCHING

## INTRODUCTION

In this lab session, we were introduced to Branching. There are three types of Jump instructions; short, near and far (long). A short JMP is the relative JMP that you refer to. It is encoded as two bytes; the actual JMP and the number of bytes +/- relative to the current IP. A near jump allows you to jump within the current "segment" (using real mode terms) or within the currently selected memory area in the CS selector. A long or far JMP additionally includes a selector (or segment in real mode).

## OBJECTIVE

The course learning objective of this lab session was

- To understand various jump instructions

- To get familiarize with Relative Addressing

- To get an idea of types of jump instructions

## PROCEDURE

Question 1

Z=0
S=0F
C=0

- Jg greater will not be taken

- Jl smaller will be taken

- Ja above will be taken

- Jb below  will not be taken

Question 2

[ORG 0x0100]

;searching 23 in the binary tree

;counting the total no of elements in the sorted array

mov DI,0

mov al,[Array+DI]

again:

inc DI

mov al,[Array+DI]

cmp al,0

jne again

mov dx,DI

;dx stores the total no of elements in the array

mov ax,DI

mov bl,2

div bl

mov cx,ax

;cx now stores the middle index of the array

;checking the midle index

mov DI,ax

mov al,[Array+DI]

mov bl,23

cmp al,bl

je equal

cmp al,bl

jg upper_class

;lower_class

mov ax,cx

mov bx,2

div bx

mov DI,ax

mov al,[Array+DI]

mov bl,23

cmp al,bl

je equal

cmp al,bl

jg upper

;lower

mov ax,DI

mov bx,2

div bx

mov DI,ax

mov al,[Array+DI]

mov bl,23

cmp al,bl

je equal

cmp al,bl

jg up

dec DI

mov ax,DI

mov bx,2

div bx

mov DI,ax

mov al,[Array+DI]

mov bl,23

cmp al,bl

je equal

mov ax,0

jmp end

up:

inc DI

mov ax,DI

mov bx,2

div bx

mov DI,ax

mov al,[Array+DI]

mov bl,23

cmp al,bl

je equal

mov ax,0

jmp end

upper_class:

sub dx,cx

mov ax,dx

mov bx,2

div bx

add ax,cx

;checking upper_class' middle index

mov DI,ax

mov al,[Array+DI]

mov bl,23

cmp al,bl

je equal

cmp al,bl

jg greater

;lesser

mov ax,DI

mov bx,cx

sub ax,bx

add ax,cx

mov DI,ax

mov al,[Array+DI]

mov bl,23

cmp al,bl

je equal

cmp al,bl

jg up

dec DI

mov ax,DI

mov bx,2

div bx

mov DI,ax

mov al,[Array+DI]

mov bl,23

cmp al,bl

je equal

mov ax,0

jmp end

greater:

mov ax,dx

mov bx,DI

sub ax,bx

add ax,DI

mov DI,ax

mov al,[Array+DI]

mov bl,23

cmp al,bl

je equal

cmp al,bl

jg up

dec DI

mov ax,DI

mov bx,2

div bx

mov DI,ax

mov al,[Array+DI]

mov bl,23

cmp al,bl

je equal

mov ax,0

jmp end

upper:

mov ax,cx

mov bx,DI

sub ax,bx

add ax,cx

mov DI,ax

mov al,[Array+DI]

mov bl,23

cmp al,bl

je equal

cmp al,bl

jg up

dec DI

mov ax,DI

mov bx,2

div bx

mov DI,ax

mov al,[Array+DI]

mov bl,23

cmp al,bl

je equal

mov ax,0

jmp end

equal:

mov ax,1

end:

MOV AX, 0x4C00 ; TERMINATE PROGRAM

INT 0x21

Array: db 2,5,8,12,16,23,38,56,72,91,0

Question 3

[ORG 0x0100]

;dx contains the factorial found of the number

mov dx,0

mov cx,1

;bl contains the no. for factorial

mov bl,5

mov al,bl

cmp al,dl

je zero_ka_factorial_one_hota_hai

dec bl

again:

MUL bl

dec bl

cmp bl,cl

jg again

mov dx,ax

jmp end

zero_ka_factorial_one_hota_hai:

mov al,1

mov dx,ax

end:

MOV AX, 0x4C00 ; TERMINATE PROGRAM

INT 0x21

## ISSUES

There were no issues.

## CONCLUSION

The relative addressing is different from absolute addressing. It does not tell the exact address rather it tells how much forward or backward to go from the current position of IP in the current code segment. We also learnt about the three different types of jumps which are the short jump that can go +127 bytes ahead in code and -128 bytes backwards and no more, the near jump that can jump anywhere within a segment and lastly the long or far jump that can go from one code segment to another.

## APPLICATIONS

Jump instructions is used to jump the memory address from one address to another wherever you want or you want to skip some line of code when your condition of statement is full fill.

## POSTLAB

No postlab.