# COMPUTER ORGANISATION AND ARCHITECTURE
LAB

FAROOQ AHMAD 								EXPERIMENT # 8
17L4407									LAB REPORT # 8
BEE-6A1

BIT MANIPULATIONS

## INTRODUCTION

In this lab session, we were introduced to Bit manipulation. Shift instructions can perform two basic types of shift operations; the logical shift and the arithmetic shift. Also, each of these operations can be performed to the right or to the left. The shifting is usually used for multiplication and division by 2, check Binary multiplier and Binary Division while arithmetic shift left is used for signed numbers, while the logical shifting is done for unsigned operations. The rotation of bits can be used for many things, there is the rotation through carry and the pure rotation. With carry is for example useful to print out the binary representation of a number, so you have a non-destructive version of getting all the bits out and into the carry. The shift instructions include SHL, SHR, SAL, and, SAR instructions. In a logical shift, the data is shifted in the appropriate direction, and a zero is shifted into the new location. In an arithmetic shift, the data is shifted right so that the sign of the data item is preserved. This means that the MSB is the value that is shifted into the new position. Rotate instructions include ROL, ROR, RCL, and, RCR instructions: The operation of these instructions is that they have the ability to rotate the contents of either an internal register or a storage location in memory. Also, the rotation that takes place can be from 1 to 255 bit positions to the left or to the right. Moreover, in the case of a multi bit rotate, the number of bit positions to be rotated is specified by the value in CL.

## OBJECTIVE

The course learning objective of this lab session was to be able to use shifting and rotation instructions in solving some problems using 8086 and to get familiarize with logical instructions.

## PROCEDURE

Question 1

[ORG 0x0100]

mov AX,1001101101010110b

ror ax,8

MOV AX, 0x4C00 ; TERMINATE PROGRAM

INT 0x21

Question 2

[ORG 0x0100]

mov AX,1100010110100010b

mov cx,16

again:

ror ax,1

jc count

back:

loop again

jmp skip

count:

inc bx

jmp back

skip:

mov ax,bx

mov bx,0

dobara:

shr al,1

jc iter

wapis:

cmp ax,1

jne dobara

jmp tapao

iter:

inc bx

jmp wapis

tapao:

MOV AX, 0x4C00 ; TERMINATE PROGRAM

INT 0x21

Question 3

[ORG 0x0100]

mov ax,0

mov bx,0

mov di,16

mov cx,[q]

mov dx,[m]

again:

mov si,1

and si,cx

cmp si,1

je yes

no:

jmp shift

yes:

add bx,dx

jc set

jmp shift

set:

mov ax,1

jmp shift

shift:

clc

rcr ax,1

rcr bx,1

rcr cx,1

clc

dec di

cmp di,0

je end_code

jmp again

end_code:

MOV AX, 0x4C00 ; TERMINATE PROGRAM

INT 0x21

q: dw 200

m: dw 256

## ISSUES

There were no issues.

## CONCLUSION

From the above experiment we can conclude that the 8086 architecture enable us to use shifting and rotation instructions. Since the registers in 8086 are not bit addressable, we can use these logical and rotation instructions to get the desired bits of any register and manipulate them according to our functions. Logical instructions of 8086 include the AND, OR, XOR, NOT type instructions. With these we can also use rotation instructions such as RCL, RCR, ROL, ROR, SAR, SHL, SHR. We can also use the logical instructions for masking such as selective bit clearing, setting and inversion.

## APPLICATIONS

Shifting and rotating bits are useful for several low level “bit twiddling” purposes, such as data compression and decompression, encryption, and device hardware register reading and control.

## POSTLAB

No postlab.