# COMPUTER ORGANISATION AND ARCHITECTURE
LAB

FAROOQ AHMAD 								EXPERIMENT # 9
17L4407									LAB REPORT # 9
BEE-6A1

SUBROUTINES AND STACK

## INTRODUCTION

In this lab session, we were introduced to the sub routines and stack. In every processor, instructions are available to divert temporarily and to divert permanently. The instructions for permanent diversion in 8088 are the jump instructions, while the instruction for temporary diversion is the CALL instruction. The stack is a block of memory that may be used for temporarily storing the contents of the registers inside the CPU. It is a top-down data structure whose elements are accessed using the stack pointer (SP) which gets decremented by two as we store a data word into the stack and gets incremented by two as we retrieve a data word from the stack back to the CPU register. It is very important to do equal number of PUSHs and POPs, otherwise the stack maybe corrupted and it will be impossible to return to operating system. We use RET instruction to return to operating system, so when program starts there is a return address in stack. CALL instruction is used to call a subroutine. Procedures or subroutines are very important in assembly language, as the assembly language programs tend to be large in size. Procedures are identified by a name. Following this name, the body of the procedure is described which performs a well-defined job. End of the procedure is indicated by a return statement.

## OBJECTIVE

The course learning objective of this lab session was

- To be able to write subroutines in assembly

- To understand basic concepts of stack

## PROCEDURE

Question 1

[ORG 0x0100]

mov ax,1

mov bx,10

mov cx,2

push ax

push bx

push cx

mov SI,0

call insert

pop cx

pop bx

pop ax

inc ax

inc cx

call insert

jmp end

insert:

mov [Array+SI],ax

again:

cmp ax,bx

jg exit

inc dx

add ax,cx

mov [Array+SI],ax

inc SI

jmp again

exit:

ret

end:

MOV AX, 0x4C00 ; TERMINATE PROGRAM

INT 0x21

Array: db 0,0,0,0,0

Question 2

Part A

[org 0x0100]

call addtoSet

call callSet

jmp end

addtoSet:

mov SI,0

mov ax,r1

mov [offsets+SI],ax

add SI,2

mov ax,r1

mov [offsets+SI],ax

add SI,2

mov ax,r1

mov [offsets+SI],ax

add SI,2

mov ax,r1

mov [offsets+SI],ax

add SI,2

mov ax,r1

mov [offsets+SI],ax

add SI,2

ret

callSet:

mov SI,0

mov ax,[offsets+SI]

call ax

add SI,2

mov ax,[offsets+SI]

call ax

add SI,2

mov ax,[offsets+SI]

call ax

add SI,2

mov ax,[offsets+SI]

call ax

add SI,2

mov ax,[offsets+SI]

call ax

add SI,2

ret

r1:

add ax,0x0001

ret

r2:

add ax,0x0002

ret

r3:

add ax,0x0003

ret

r4:

add ax,0x0004

ret

r5:

add ax,0x0005

ret

end:

mov ax, 0x4c00 ; terminate program

int 0x21

offsets: dw 0,0,0,0,0

Question 2

Part B

[org 0x0100]

call addtoSet

call callSet

jmp end

addtoSet:

mov SI,0

mov ax,r1

mov [offsets+SI],ax

add SI,2

mov ax,r1

mov [offsets+SI],ax

add SI,2

mov ax,r1

mov [offsets+SI],ax

add SI,2

mov ax,r1

mov [offsets+SI],ax

add SI,2

mov ax,r1

mov [offsets+SI],ax

add SI,2

ret

callSet:

mov ax,[offsets+8]

push ax

mov ax,[offsets+6]

push ax

mov ax,[offsets+4]

push ax

mov ax,[offsets+2]

push ax

mov ax,[offsets+0]

push ax

ret

r1:

add ax,0x0001

ret

r2:

add ax,0x0002

ret

r3:

add ax,0x0003

ret

r4:

add ax,0x0004

ret

r5:

add ax,0x0005

ret

end:

mov ax, 0x4c00 ; terminate program

int 0x21

offsets: dw 0,0,0,0,0

## ISSUES

There were no issues.

## CONCLUSION

The stack segment register (SS) is usually used to store information about the memory segment that stores the call stack of currently executed program. Subroutines are often used to perform tasks that need to be performed frequently and a CALL instruction can be used to execute subroutines. The RET returns to the address stored at the top of the stack.

## APPLICATIONS

Stack is widely used in carrying out tasks such as expression evaluation, expression conversion, syntax parsing, backtracking, parenthesis checking, string reversal and function calls.