# Data Communication and Network
LAB

FAROOQ AHMAD									Experiment # 4
17L4407										Lab Report # 4
BEE-5A

## BUILDING TOPOLOGIES IN NETWORK SIMULATOR VERSION-3 
(NS-3)

## INTRODUCTION

In this lab session, we were introduced to building up a network topology in network simulator version 3 (ns 3) developing a point to point link topology between two nodes and extend the point to point link by hanging a bus network off of the right side. This can be done by implementing CSMA channel. Moreover the extension is going to be extended to WIFI network. A new network device to the node on the left side of the point-to-point link that becomes the access point for the wireless network. A number of wireless STA nodes are created to fill out the new network. A real Ethernet uses CSMA/CD (Carrier Sense Multiple Access with Collision Detection) scheme with exponentially increasing back off to contend for the shared transmission medium. The ns-3 CSMA device and channel models only a subset of this.

## OBJECTIVE

The course learning objective of this lab session was to introduce us to multi network topology point to point connection between two devices via router module. The processing objective is to build and analyze network topology using CSMA channel.

## DESIGN PROCEDURE

Create simple point to point topology between two nodes.
Extend point-to-point example (the link between the nodes n0 and n1 below) by hanging a bus network off of the right side.
 This can be done by implement CSMA channel. Our task is to create four nodes on LAN (CSMA channel) – one required node and three extra nodes.

/* -*- Mode:C++; c-file-style:"gnu"; indent-tabs-mode:nil; -*- */

/*

* This program is free software; you can redistribute it and/or modify

* it under the terms of the GNU General Public License version 2 as

* published by the Free Software Foundation;

*

* This program is distributed in the hope that it will be useful,

* but WITHOUT ANY WARRANTY; without even the implied warranty of

* MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE.  See the

* GNU General Public License for more details.

*

* You should have received a copy of the GNU General Public License

* along with this program; if not, write to the Free Software

* Foundation, Inc., 59 Temple Place, Suite 330, Boston, MA  02111-1307  USA

*/

#include "ns3/core-module.h"

#include "ns3/network-module.h"

#include "ns3/csma-module.h"

#include "ns3/internet-module.h"

#include "ns3/point-to-point-module.h"

#include "ns3/applications-module.h"

#include "ns3/ipv4-global-routing-helper.h"

// Default Network Topology

//

//       10.1.1.0

// n0 -------------- n1   n2   n3   n4

//    point-to-point  |    |    |    |

//                    ================

//                      LAN 10.1.2.0

using namespace ns3;

NS_LOG_COMPONENT_DEFINE ("SecondScriptExample");

int

main (int argc, char *argv[])

{

bool verbose = true;

uint32_t nCsma = 3;

CommandLine cmd;

cmd.AddValue ("nCsma", "Number of \"extra\" CSMA nodes/devices", nCsma);

cmd.AddValue ("verbose", "Tell echo applications to log if true", verbose);

cmd.Parse (argc,argv);

if (verbose)

{

LogComponentEnable ("UdpEchoClientApplication", LOG_LEVEL_INFO);

LogComponentEnable ("UdpEchoServerApplication", LOG_LEVEL_INFO);

}

nCsma = nCsma == 0 ? 1 : nCsma;

NodeContainer p2pNodes;

p2pNodes.Create (2);

NodeContainer csmaNodes1,csmaNodes2;

csmaNodes1.Add (p2pNodes.Get (1));

csmaNodes1.Create (nCsma);

csmaNodes2.Add (p2pNodes.Get (0));

csmaNodes2.Create (nCsma);

PointToPointHelper pointToPoint;

pointToPoint.SetDeviceAttribute ("DataRate", StringValue ("5Mbps"));

pointToPoint.SetChannelAttribute ("Delay", StringValue ("2ms"));

NetDeviceContainer p2pDevices;

p2pDevices = pointToPoint.Install (p2pNodes);

CsmaHelper csma;

csma.SetChannelAttribute ("DataRate", StringValue ("100Mbps"));

csma.SetChannelAttribute ("Delay", TimeValue (NanoSeconds (6560)));

NetDeviceContainer csmaDevices1,csmaDevices2;

csmaDevices1 = csma.Install (csmaNodes1);

csmaDevices2 = csma.Install (csmaNodes2);

InternetStackHelper stack;

stack.Install (csmaNodes1);

stack.Install (csmaNodes2);

Ipv4AddressHelper address;

address.SetBase ("10.1.1.0", "255.255.255.0");

Ipv4InterfaceContainer p2pInterfaces;

p2pInterfaces = address.Assign (p2pDevices);

address.SetBase ("10.1.2.0", "255.255.255.0");

Ipv4InterfaceContainer csmaInterfaces1;

csmaInterfaces1 = address.Assign (csmaDevices1);

address.SetBase ("10.1.3.0", "255.255.255.0");

Ipv4InterfaceContainer csmaInterfaces2;

csmaInterfaces2 = address.Assign (csmaDevices2);

UdpEchoServerHelper echoServer (93);

ApplicationContainer serverApps = echoServer.Install (csmaNodes1.Get (2));

serverApps.Start (Seconds (1.0));

serverApps.Stop (Seconds (10.0));

UdpEchoClientHelper echoClient (csmaInterfaces1.GetAddress (2), 93);

echoClient.SetAttribute ("MaxPackets", UintegerValue (1));

echoClient.SetAttribute ("Interval", TimeValue (Seconds (1.0)));

echoClient.SetAttribute ("PacketSize", UintegerValue (1024));

ApplicationContainer clientApps = echoClient.Install (csmaNodes2.Get (2));

clientApps.Start (Seconds (2.0));

clientApps.Stop (Seconds (10.0));

Ipv4GlobalRoutingHelper::PopulateRoutingTables ();

pointToPoint.EnablePcapAll ("second");

csma.EnablePcap ("second", csmaDevices.Get (1), true);

Simulator::Run ();

Simulator::Destroy ();

return 0;

}

## ISSUES

There were as such none issues.

## CONCLUSION

This lab session was conducted successfully. The concepts of network simulator 3 were understood and used to build and analyze a network topology using point to point and CSMA channel. The network topology using point to point link between two nodes was build and analyzed with the help of CSMA channel.

## APPLICATIONS

NS3 helps to create various virtual nodes (i.e., computers in real life) and with the help of various Helper classes it allows us to install devices, internet stacks, application, etc to our nodes.
Topology is used in many branches of mathematics, such as differentiable equations, dynamical systems, knot theory, and Riemann surfaces in complex analysis. It is also used in string theory in physics, and for describing the space-time structure of universe.
Using NS3 we can create PointToPoint, Wireless, CSMA, etc connections between nodes. PointToPoint connection is same as a LAN connected between two computers. Wireless connection is same as WiFi connection between various computers and routers.

## POST LAB

/* -*- Mode:C++; c-file-style:"gnu"; indent-tabs-mode:nil; -*- */

/*

* This program is free software; you can redistribute it and/or modify

* it under the terms of the GNU General Public License version 2 as

* published by the Free Software Foundation;

*

* This program is distributed in the hope that it will be useful,

* but WITHOUT ANY WARRANTY; without even the implied warranty of

* MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE.  See the

* GNU General Public License for more details.

*

* You should have received a copy of the GNU General Public License

* along with this program; if not, write to the Free Software

* Foundation, Inc., 59 Temple Place, Suite 330, Boston, MA  02111-1307  USA

*/

#include "ns3/core-module.h"

#include "ns3/point-to-point-module.h"

#include "ns3/network-module.h"

#include "ns3/applications-module.h"

#include "ns3/wifi-module.h"

#include "ns3/mobility-module.h"

#include "ns3/csma-module.h"

#include "ns3/internet-module.h"

// Default Network Topology

//

//   Wifi 10.1.3.0

//                 AP

//  *    *    *    *

//  |    |    |    |    10.1.1.0

// n5   n6   n7   n0 -------------- n1   n2   n3   n4

//                   point-to-point  |    |    |    |

//                                   ================

//                                     LAN 10.1.2.0

using namespace ns3;

NS_LOG_COMPONENT_DEFINE ("ThirdScriptExample");

int

main (int argc, char *argv[])

{

bool verbose = true;

uint32_t nWifi = 3;

CommandLine cmd;

cmd.AddValue ("nWifi", "Number of wifi STA devices", nWifi);

cmd.AddValue ("verbose", "Tell echo applications to log if true", verbose);

cmd.Parse (argc,argv);

if (nWifi > 18)

{

std::cout << "Number of wifi nodes " << nWifi <<

" specified exceeds the mobility bounding box" << std::endl;

exit (1);

}

if (verbose)

{

LogComponentEnable ("UdpEchoClientApplication", LOG_LEVEL_INFO);

LogComponentEnable ("UdpEchoServerApplication", LOG_LEVEL_INFO);

}

NodeContainer p2pNodes;

p2pNodes.Create (2);

PointToPointHelper pointToPoint;

pointToPoint.SetDeviceAttribute ("DataRate", StringValue ("5Mbps"));

pointToPoint.SetChannelAttribute ("Delay", StringValue ("2ms"));

NetDeviceContainer p2pDevices;

p2pDevices = pointToPoint.Install (p2pNodes);

NodeContainer wifiStaNodes1,wifiStaNodes2;

wifiStaNodes1.Create (nWifi);

wifiStaNodes2.Create (nWifi);

NodeContainer wifiAp1Node = p2pNodes.Get (0);

NodeContainer wifiAp2Node = p2pNodes.Get (1);

YansWifiChannelHelper channel = YansWifiChannelHelper::Default ();

YansWifiPhyHelper phy = YansWifiPhyHelper::Default ();

phy.SetChannel (channel.Create ());

WifiHelper wifi = WifiHelper::Default ();

wifi.SetRemoteStationManager ("ns3::AarfWifiManager");

NqosWifiMacHelper mac = NqosWifiMacHelper::Default ();

Ssid ssid = Ssid ("ns-3-ssid");

mac.SetType ("ns3::StaWifiMac",

"Ssid", SsidValue (ssid),

"ActiveProbing", BooleanValue (false));

NetDeviceContainer staDevices1,staDevices1;

staDevices1 = wifi.Install (phy, mac, wifiStaNodes1);

staDevices2 = wifi.Install (phy, mac, wifiStaNodes2);

mac.SetType ("ns3::ApWifiMac",

"Ssid", SsidValue (ssid));

NetDeviceContainer apDevices1,apDevices2;

apDevices1 = wifi.Install (phy, mac, wifiAp1Node);

apDevices2 = wifi.Install (phy, mac, wifiAp2Node);

MobilityHelper mobility;

mobility.SetPositionAllocator ("ns3::GridPositionAllocator",

"MinX", DoubleValue (0.0),

"MinY", DoubleValue (0.0),

"DeltaX", DoubleValue (5.0),

"DeltaY", DoubleValue (10.0),

"GridWidth", UintegerValue (3),

"LayoutType", StringValue ("RowFirst"));

mobility.SetMobilityModel ("ns3::RandomWalk2dMobilityModel",

"Bounds", RectangleValue (Rectangle (-50, 50, -50, 50)));

mobility.Install (wifiStaNodes1);

mobility.Install (wifiStaNodes2);

mobility.SetMobilityModel ("ns3::ConstantPositionMobilityModel");

mobility.Install (wifiAp1Node);

mobility.Install (wifiAp2Node);

InternetStackHelper stack;

stack.Install (wifiAp1Node);

stack.Install (wifiStaNodes1);

stack.Install (wifiAp2Node);

stack.Install (wifiStaNodes2);

Ipv4AddressHelper address;

address.SetBase ("10.1.1.0", "255.255.255.0");

Ipv4InterfaceContainer p2pInterfaces;

p2pInterfaces = address.Assign (p2pDevices);

address.SetBase ("10.1.2.0", "255.255.255.0");

address.Assign (staDevices1);

address.Assign (apDevices1);

address.SetBase ("10.1.3.0", "255.255.255.0");

address.Assign (staDevices2);

address.Assign (apDevices2);

UdpEchoServerHelper echoServer (93);

ApplicationContainer serverApps = echoServer.Install (wifiStaNodes1.Get (2));

serverApps.Start (Seconds (1.0));

serverApps.Stop (Seconds (10.0));

UdpEchoClientHelper echoClient (wifiStaNodes1.GetAddress (2), 9);

echoClient.SetAttribute ("MaxPackets", UintegerValue (1));

echoClient.SetAttribute ("Interval", TimeValue (Seconds (1.0)));

echoClient.SetAttribute ("PacketSize", UintegerValue (1024));

ApplicationContainer clientApps =

echoClient.Install (wifiStaNodes2.Get (2));

clientApps.Start (Seconds (2.0));

clientApps.Stop (Seconds (10.0));

Ipv4GlobalRoutingHelper::PopulateRoutingTables ();

Simulator::Stop (Seconds (10.0));

pointToPoint.EnablePcapAll ("third");

phy.EnablePcap ("third", apDevices.Get (0));

csma.EnablePcap ("third", csmaDevices.Get (0), true);

Simulator::Run ();

Simulator::Destroy ();

return 0;

}