| Course Title | Embedded Systems | Course Code | EE419 |
| --- | --- | --- | --- |
| Pre-requisite(s) | Microprocessor Programming and Interfacing (EE-303) | Credit Hrs | 3 |

# ASSIGNMENT 2

| No. | Assigned Program Learning Outcome (PLO) | Level | Tool |
| --- | --- | --- | --- |
| 05 | An ability to create, select and apply appropriate techniques, resources, and modern engineering and IT tools, including prediction and modeling, to complex engineering activities, with an understanding of the limitations. | R | A2 |
| I = Introduction, R = Reinforcement, E = Evaluation. A = Assignment, Q = Quiz, M = Midterm, F=Final, L=Lab, P=Project, W=Written Report. |  |  |  |

# DEPARTMENT OF ELECTRICAL ENGINEERING
FALL 2020

GROUP PARTNERS 						SECTION BEE-7B

- FAROOQ AHMAD					PROFESSOR:
17L-4407   							Ms. TOOBA JAVED

- NAYAB AHMAD
17L-4498

Problem Statement:

We want to design a home automation system using TM4C Controller. This system detects an intruder, abnormal temperature condition inside a house and any smoke that is hazardous to human life. On detecting any abnormal activity, an alarm should be turned on for few seconds to notify the resident. The system sends status of each sensor continuously to a PC connected with it. User can control devices via PC as well. For example, user can turn off the alarm from PC. Your system should not cost more than PKR 2000.

Solution:

# PLO 5.1

## 5.1.1 Decide which devices should be used as inputs and output for this system

This system has to take different inputs for doing different tasks and then has to give different outputs thus different input and output devices would be used. After looking at the requirements of the system, we short listed the input and output devices that will be used in the home automation system.

| Device | Type | Purpose |
| --- | --- | --- |
| TM4C123GH6PM | Input/ Output | Processing of input data & give output |
| Intruder Sensor | Input | Detection of any intruder |
| Smoke Sensor | Input | Detection of harmful smoke |
| Temperature Sensor | Input | Detection of abnormal temperature |
| Buzzer | Output | Alarm if any abnormal activity detected |

## 5.1.2 Identify an efficient method/technique of interfacing these devices with microcontroller

Interfacing can be defined as transferring data between microcontrollers and (interfacing  peripherals such as) sensors, keypads, microprocessors, analog to digital converters or ADC, LCD  displays, motors, external memories, even with other microcontrollers, some other interfacing peripheral devices and so on or input devices and output devices.

For the purpose of micro processing unit, we will be using microcontroller TM4C. This would be acting as the central unit for taking inputs, processing and sending outputs. I/O pins on TIVA microcontrollers have a wide range of alternative functions:

- UART Universal asynchronous receiver/transmitter

- SSI Synchronous serial interface

- I2C Inter-integrated circuit

- Timer Periodic interrupts, input capture, and output compare

- PWM Pulse width modulation

- ADC Analog to digital converter, measure analog signals

Now each sensor i.e. intruder sensor, smoke sensor and temperature would be interfaced with the microcontroller with respective type of pins.

- First of all, intruder sensor is a digital sensor. Thus, it can be interfaced with any of the GPIO pins. So, we used port F for connecting this sensor and microcontroller.

- Then for temperature sensor, we can use analog to digital converter of this microcontroller. As this module has two analogs to digital converter modules i.e. 0 and 1 so any one can be used for interfacing this sensor. The module ADC0 is used for connecting temperature sensor to microcontroller. These modules are present at port D.

- Similarly, the smoke sensor can be connected to the analog to digital converter module. So, it can be interfaced with ADC1 module.

- For displaying the status on the PC, we can use the Universal asynchronous receiver/transmitter (UART). We used UART module 0 which is present at port A.

- Buzzer is an output module which can be turned on by sending pulses to it. For producing waves, we can use Pulse Width Modulation module (PWM). There are also number of modules of this type. This module is attached to port E. Thus, we connected the buzzer to port E.

## 5.1.3 Mention all the software tools used

The software was needed to interface all these devices and make one whole system.

▪ Keil uvision (version 5) was used to do programming. The µVision IDE combines project management, run-time environment, build facilities, source code editing, and program debugging in a single powerful environment. µVision is easy-to-use and accelerates your embedded software development.

▪ C language was used for coding. This had to be done on tm4c but we did not perform this project in the lab, but rather in our homes where we did not have access to such modules.

▪ Tera Term is the software which is used to display the output through PC. Tera Term is an open-source, free, software implemented, and terminal emulator program. But since we don’t have access to microcontrollers we couldn’t perform our task on tera term.

# PLO 5.2

## 5.2.1 Compare different sensors for intrusion, temperature and gas detection and chose the best for your system

Now for finalizing the components we short listed different characteristics and on their basis, we selected the best suited components for our project.

Intrusion Detection Sensor

For detection of the intruder following sensors were kept in view.

| Sensor | Principle | Pros | Cons |
| --- | --- | --- | --- |
| Infrared Light Barriers | Ray of light between receiver and transmitter is interrupted. | ▪ Affordable ▪ Practical ▪ Generate their own energy ▪ Covers wide range of area | ▪ Cannot pass through glass ▪ Cannot be used in outside environment |
| Ultrasonic Sensors | Emit ultrasonic sound energy into monitored area and reacts to a change in reflected energy pattern. | ▪ Affordable ▪ Not affected by outside environment | ▪ Excessive air motion can trigger it |
| Microwave Detectors | Changes caused by intruders in the electromagnetic field between the transmitter and receiver are detected | ▪ Weather insensitive ▪ Extremely reliable if installed properly | Special care is required when installing these sensors |
| Magnetic Switches | Switch detects change in magnetic field | ▪ Measurement range is very small |  |

Looking at the above characteristics, infrared sensor seemed most suitable for our home automation system. So, we selected infrared sensor of MH sensor series.

Smoke Detection Sensor

Before smoke detectors were introduced in the market, heat sensors were used for detection of fire. But now smoke sensors are present solely for the purpose of detection of different kinds of smoke. Keeping in view the budget limitation, we picked the most cost-efficient smoke sensor present easily in market i.e. MQ-2.

Gas Sensor MQ-2 module is useful for gas leakage detection in home and industry. It can detect H2, LPG, CH4, CO, Alcohol, Smoke, and Propane. Following are the most prominent features of this sensor:

- Good sensitivity to combustible gas in wide range thus quick and easy detection of hazardous gas.

- High sensitivity to LPG, Propane and Hydrogen which helped us in testing our system as LPG was easily available.

- Long life and low cost which was the requirement of our project.

Temperature Detection Sensor

For temperature detection, we took LM35 temperature sensor as we have already done experiments and projects using this sensor. It is a precision integrated-circuit temperature sensing device with an output voltage linearly proportional to centigrade temperature.

- The LM35 device has an advantage over linear temperature sensors calibrated in Kelvin as the user is not required to subtract a large constant voltage from the output to obtain convenient Centigrade scaling.

- It is a 3 terminal sensor used to measure surrounding temperature ranging from -55 °C to 150 °C.

- It also fulfills the cost efficiency requirement of our project thus it was finalized as the module for detecting abnormal temperature.

- LM35 gives temperature output which is more precise than thermistor output.

# PLO 5.3

## 5.3.1 Draw a hardware block diagram of your system with all the chosen components

## 5.3.2 Draw flow chart of your software

## 5.3.3 List all the on chip modules you have used

The following modules are used in our home automation system.

▪ Universal asynchronous receiver/transmitter (UART) – Module 0

UART0 module was used because its serial connections of port A are already done with PC through USB port.

▪ Pulse Width Modulation (PWM) - Module 0

Out of two modules of PWM, module 0, generator 1 and output pin A (PE4) was used to  generate 1kHz wave of 50% duty cycle for configuring the buzzer.

▪ Analog to Digital Converter (ADC) – Module 0 & 1

Both of the ADC modules are configured as their FIFO depth is 1, sequencer is 3, trigger option is processor based and ADC6 (PD1) and ADC7 (PD0) are used for temperature and Smoke sensor respectively.

## 5.3.4 Write an efficient code for your system with minimum wastage of CPU resources

#include <stdbool.h>

#include <stdio.h>

#include <stdint.h>

#include <string.h>

#include "C:\ti\TivaWare_C_Series-2.1.4.178\inc\tm4c123gh6pm.h"

volatile char temp1 = 'A';

bool flag=false;

bool flagb=false;

bool buzoff =false;

float f;

int e;

int temp;

int result=0;

void EnableInterrupts(void)

{

__asm("cpsie i\n");

}

void UART0Tx(char c)

{

UART0_DR_R = c;

}

void Buzzer()//USING PE4

{

SYSCTL_RCGCPWM_R |=2; //Enabling clock to PWM1

SYSCTL_RCGCGPIO_R|=0x10; //Enabling clock to PortE

SYSCTL_RCC_R &=~0x00100000; //No clock pre-divider for PWM

GPIO_PORTE_AFSEL_R=0x10; //Alternate Function Enable for PE4 GPIO_PORTE_PCTL_R = 0x50000;//Alternate Function value for PE4

GPIO_PORTE_DIR_R |=0x10; //PE4 as output

GPIO_PORTE_DEN_R |=0x10; //Enabling Digital at PE4

PWM1_1_CTL_R = 0; //Stop Counter PWM1_1_GENA_R = 0x8C; //Output Set when reload and clear when matches CMPA PWM1_1_LOAD_R = 16000; //for 1KHz wave

PWM1_1_CMPA_R = 8000; //50% duty cycle

PWM1_1_CTL_R = 1; //Start Counter PWM1_ENABLE_R |= 0x4; //Enable PWM

}

voidGPIO_Int_En(void)

{

GPIO_PORTF_IM_R |=0x1; //Enabling intruppt on PF0

NVIC_EN0_R |=(1<<30); //IRQ of PORT F

GPIO_PORTF_IS_R &= ~(0x1); //Edge triggered on PF0

GPIO_PORTF_IEV_R |= 0x1; // Rising edge

}

void Uart_Int_En(void)

{

NVIC_EN0_R |=(1<<5);//IRQ of Port A

UART0_IM_R |= 0x00030; //Rx Tx intruppt enable

}

void PortF_Init(void)

{

SYSCTL_RCGCGPIO_R |= 0x00000020; // activate clock for Port F

while((SYSCTL_PRGPIO_R&0x00000020) == 0){};// ready?

GPIO_PORTF_LOCK_R = 0x4C4F434B; // unlock GPIO Port F

GPIO_PORTF_CR_R |= 0x01; //Allow changes at PF0

GPIO_PORTF_DIR_R &= (~0x01); //Input at PF0

GPIO_PORTF_PUR_R |= 0x01; //Pull Up register at PF0

GPIO_PORTF_DEN_R |= 0x01; //Digital enable at PF0

}

void delay()

{

uint32_t counter=0;

for (counter=0; counter<1500;counter++);

}

void UART_Init(void)

{

SYSCTL_RCGCUART_R |= 0x0001; // activate UART0

SYSCTL_RCGCGPIO_R |= 0x0001; // activate port A

UART0_CTL_R &= ~0x0001; // disable UART

UART0_IBRD_R = 8; //Decimal part of Baud rate

UART0_FBRD_R = 44; //Fractional part of Baud Rate

UART0_LCRH_R = 0x0060; // 8-bit length, disable FIFO

UART0_CTL_R = 0x0301; // enable RXE, TXE and UART

GPIO_PORTA_AFSEL_R |= 0x03; // alt funct on PA1-0

GPIO_PORTA_PCTL_R = 0x00000011; //Alternate function value

GPIO_PORTA_DEN_R |= 0x03; // digital I/O on PA1-0

GPIO_PORTA_DIR_R=2; //Output on PA1-0

GPIO_PORTA_AMSEL_R &= ~0x03; // No analog on PB1-0

}

void UART0_Handler(void)

{

if (UART0_MIS_R & 0x0010) //check on recieve flag

{

temp1=UART0_DR_R; //store data in temp1

UART0_ICR_R=0x0010; //Clearing recieving flag

if(temp1=='b')

{

PWM1_ENABLE_R &= ~0x4; //disable PWM

buzoff=true; //buzzer flag

}

}

else

UART0_ICR_R=0x0020; //check on transmit flag

}

void sendString(char*ptr)

{

int i;

for(i=0;i<strlen(ptr);i++)

{

UART0Tx(ptr[i]); //sending string

delay();

}

}

void GPIOPortF_Handler(void)

{

sendString("\r*............INTRUDER DETECTED..........*\r");

Buzzer();

for(e=0;e<10000000;e++);

GPIO_PORTF_ICR_R|=1; //clearing flag

}

void ADC0Seq3_Handler(void) //Temperature Intruppt

{

result = ADC0_SSFIFO3_R;

f=(result*330)/4096;

if(f>70)

{

if(buzoff==false)

Buzzer();

sendString(" .........*TEMPERATURE IS

HIGH*................\r");

flag=true;

}

else

{

if(flagb==false)

{

buzoff=false;

}

flag=false;

}

temp=f;

sendString("Temperature = ");

delay();

if(temp/100!=0)

{ UART0Tx((temp/100)+48);

delay();

temp%=100;

}

UART0Tx(temp/10+48);

delay();

UART0Tx(temp%10+48);

delay();

sendString("'C ");

delay();

ADC0_ISC_R=8;

}

void ADC1Seq3_Handler(void)//Smoke Intruppt {

result = ADC1_SSFIFO3_R;

f=(result*330)/4096;

temp=f;

sendString("Smoke = ");

delay();

if(temp/100!=0)

{ UART0Tx((temp/100)+48);

delay();

temp%=100;

}

UART0Tx(temp/10+48);

delay();

//UART0Tx(46);

UART0Tx(temp%10+48);

delay();

//UART0Tx(0x85);

//sendString(" \r");

if(f>200)

{

if(buzoff==false)

Buzzer();

sendString(".......*HAZARDOUS SMOKE DETECTED*..........\r");

flagb=true;

}

else

{

if(flag==false)

{

sendString(" \r");

buzoff=false;

}

flagb=false;

}

//UART0Tx('\r');

sendString("\r");

ADC1_ISC_R=8;

for(e=0;e<1000000;e++);

}

void ADC_Init(void)

{

SYSCTL_RCGCGPIO_R |= 0x8; //Unlocking Port D

SYSCTL_RCGCADC_R |=3; //Enabling ADC Module 0 & 1

GPIO_PORTD_LOCK_R = 0x4C4F434B; //Unlock

GPIO_PORTD_AFSEL_R |=3; //Alternate function for PD1-0

GPIO_PORTD_DEN_R &=~3; //Digital Disable PD1-0

GPIO_PORTD_AMSEL_R |=3; //Analog Enable PD1-0

ADC0_ACTSS_R &=~8; //Disable ADC0 seq3

ADC0_EMUX_R&=~0xF000; //Use default processor

ADC0_SSMUX3_R=6; // ADC input 6 at PD1

ADC0_SSCTL3_R |=6; //take one sample at a time, set flag at 1st sample

ADC0_ACTSS_R |=8; //Enable ADC0 seq3

NVIC_EN0_R |= (1<<17); //IRQ of ADC0 SS3

ADC0_IM_R |=8; //sample Sequencer 3

ADC1_ACTSS_R &=~8; //Disable ADC1 seq3

ADC1_EMUX_R&=~0xF000; //Use default processor

ADC1_SSMUX3_R=7; //ADC input 7 at PD0

ADC1_SSCTL3_R |=6; //take one sample at a time, set flag at 1st sample ADC1_ACTSS_R |=8; //Enable ADC1 seq3

NVIC_EN1_R |= (1<<19); //IRQ of ADC1 SS3

ADC1_IM_R |=8; //sample Sequencer 3

}

int main(void)

{

EnableInterrupts();

UART_Init();

Uart_Int_En();

PortF_Init();

GPIO_Int_En();

ADC_Init();

while(1)

{

ADC0_PSSI_R |=8;

ADC1_PSSI_R |=8;

}

}

# PLO 5.4

## 5.4.1 Simulate your code in software before testing it on hardware. If it is not working, improve your solution

The written code was tested on software using hard values. Uvision software was used for this assignment only. Since we had no access to Tera Term software & TM4C micro controller and had no proper hardware equipment to implement on with the microcontroller we weren’t able to properly detect or observe our sensors output.

# PLO 5.5

| Limitations | Recommendations |
| --- | --- |
| The detecting range of used infrared is very short i.e. 8mm. | Use expensive sensor to improve the detecting range. |
| Smoke senor MQ-2 can only detect few of the harmful smokes i.e. LPG, Propane, Methane etc. | More advanced sensor can be used to detect wide range of types of smoke e.g. Phosgene, Nitrogen Dioxide. |
| Temperature sensor LM35 has a specific range. | Wider range sensor can be used. |