# EMBEDDED SYSTEMS LAB

FAROOQ AHMAD 								EXPERIMENT # 7
17L-4407									LAB REPORT # 7
EL-7B

### SYNCHRONOUS SERIAL INTERFACE

### INTRODUCTION

In this lab session, we were introduced to the SYNCHRONOUS SERIAL INTERFACE. As per the nomenclature of the TM4C123GH6PM microcontroller, the name used for SPI modules is a synchronous serial interface (SSI). But it is popularly referred to as a serial peripheral interface. That is why we use the term SPI instead of SSI. TM4C123GF6PM microcontroller contains four serial peripheral interfaces or SPI modules. Each module can be configured in master or slave mode to receive or send data to other devices that communicate over SPI interface. SSI modules convert serial data into parallel data.

Followings are the main features of TM4C123 SPI modules:

- Master and Slave Configuration

- FIFOs for each SPI transmitter and receiver

- Direct SPI data transfer to DMA (direct memory access controller)

The fundamental difference between a UART, which implements an asynchronous protocol, and a SSI, which implements asynchronous protocol, is the manner in which the clock is implemented. Two devices communicating with asynchronous serial interfaces (UART) operate at the same frequency (baud rate) but have two separate clocks. With a UART protocol, the clock signal is not included in the interface cable between devices. Two devices communicating with synchronous serial interfaces (SSI) operate from the same clock (synchronized). With a SSI protocol, the clock signal is included in the interface cable between devices. Typically, the master device creates the clock, and the slave device(s) uses the clock to latch the data (in or out.)

### OBJECTIVE

The course learning objective of this lab session was to understand concept of synchronous communication and to learn how to configure SSI module of TM4C.

### PROCEDURE

After the GPIO configuration, we need to take the following steps to configure the SSI for the SPI protocol:

1. Enable the clock to SSI module using RCGCSSI.

2. Disable the SSI via bit 1 of SSICR1 register before initialization.

3. Set the Bit Rate with the SSICPSR pre scaler and SSICR0 control registers.

4. Also select the SPI mode, phase, polarity, and data size in SSICR0 control register.

5. Select the master mod in SSISCR1 register.

6. Enable SSI using SSICR1 register.

7. Assert slave select signal.

8. Wait until the TNF flag (transmit FIFO not full) in SSISR (status register) goes high, then load a byte of data into

SSIDR (data register) to be transmitted.

9. Repeat step 8 above until all the data are in the FIFO.

10. Wait until transmit is complete (transmit FIFO empty and SSI not busy).

11. Deassert the slave select signal.

MASTER CODE:

#include "tm4c123gh6pm.h"

void delayMs(int n);

void init_SSI2(void)

{

SYSCTL->RCGCSSI |= 4;       /* enable clock to SSI2 */

SYSCTL->RCGCGPIO |= 2;      /* enable clock to GPIOB for SSI2 */

while((SYSCTL->PRGPIO &0x0002) == 0){};// ready?

/* configure PORTB 4,5,6,7*/

GPIOB->AMSEL &= ~0xF0;      /* disable analog for these pins */

GPIOB->DEN |= 0xF0;         /* and make them digital */

GPIOB->DIR |= 0xB0;         /* and make them digital */

GPIOB->AFSEL |= 0xF0;       /* enable alternate function */

GPIOB->PCTL |= 0x22220000;/* assign pins to SSI2 */

GPIOB->DATA |= 0x20   ;        /* keep SS idle high */

/* SPI Master, SPO = 0, SPH = 0, clock = 16 MHz, 8 bit data */

SSI2->CR1 = 0;          /* disable SSI and make it master */

SSI2->CR0 = 0x00003307;     /* 9600 SSI clock, SPI mode, 8 bit data, SCR=0x04 */

SSI2->CC = 0;           /* use system clock */

SSI2->CPSR = 0x20;         /* prescaler divided by 0x20 */

SSI2->CR1 |= 2;         /* enable SSI2 */

}

void uartINnit()

{

SYSCTL->RCGCUART |=1;

SYSCTL->RCGCGPIO |=1;

//Now we will initialize the Uart0

UART0->CTL=0;

UART0->IBRD=8;

UART0->FBRD=44;

UART0->CC=0;

UART0->LCRH=0X60;

UART0->CTL=0X301;

//SETTING UP THE PINS

GPIOA->DEN=0X03;

GPIOA->AFSEL=0X03;

GPIOA->PCTL=0X11;

}

void UART0Tx(char c)

{

while((UART0->FR & 0x0020) !=0){}

UART0->DR = c;

}

void SSI2Write(unsigned char data)

{

GPIOB->DATA &= ~0x20;       /* assert SS low */

SSI2->DR = data;            /* transmit high byte */

while(SSI2->SR & 0x10);     /* wait until transmit complete */

GPIOB->DATA |= 0x20;        /* keep SS idle high */

}

char SSI2Read()

{

char data;

while(SSI2->SR & 0x10);

data = SSI2->DR ;

return data;

}

int main(void)

{

uartINnit();

init_SSI2();

while(1)

{

delay(2000);

SSI2Write('A');

char value=SSI2Read();

UART0Tx(value);

}

}

/* delay n milliseconds (16 MHz CPU clock) */

void delayMs(int n)

{

int i, j;

for(i = 0 ; i < n; i++)

for(j = 0; j < 3180; j++)

{} /* do nothing for 1 ms */

}

void SystemInit()

{

SCB->CPACR|=0XF00000;

}

SLAVE CODE:

#include "tm4c123gh6pm.h"

char value;

void uartINnit()

{

SYSCTL->RCGCUART |=1;

SYSCTL->RCGCGPIO |=1;

//Now we will initialize the Uart0

UART0->CTL=0;

UART0->IBRD=8;

UART0->FBRD=44;

UART0->CC=0;

UART0->LCRH=0X60;

UART0->CTL=0X301;

//SETTING UP THE PINS

GPIOA->DEN=0X03;

GPIOA->AFSEL=0X03;

GPIOA->PCTL=0X11;

}

void PortF_Init(void)

{

SYSCTL_RCGCGPIO_R |= 0x00000020; // activate clock for Port F

GPIOF->DIR = 0x0E; // PF4 in, PF2 out

GPIOF->DEN = 0x0E; // enable digital I/O on PF2 & PF4

}

void init_SSI2(void)

{

SYSCTL->RCGCSSI |= 4;       /* enable clock to SSI2 */

SYSCTL->RCGCGPIO |= 2;      /* enable clock to GPIOB for SSI2 */

while((SYSCTL->PRGPIO &0x0002) == 0){};// ready?

/* configure PORTB 4,5,6,7*/

GPIOB->AMSEL &= ~0xF0;      /* disable analog for these pins */

GPIOB->DEN |= 0xF0;         /* and make them digital */

GPIOB->DIR |= 0xB0;         /* and make them digital */

GPIOB->AFSEL |= 0xF0;       /* enable alternate function */

GPIOB->PCTL |= 0x22220000;/* assign pins to SSI2 */

SSI2->CR1 = 0x04;          /* disable SSI and make it slave */

SSI2->CR0 = 0x00003307;     /* 9600 SSI clock, SPI mode, 8 bit data, SCR=0x04 */

SSI2->CC = 0;           /* use system clock */

SSI2->CPSR = 0x20;         /* prescaler divided by 0x20 */

SSI2->CR1 |= 2;         /* enable SSI1 */

}

void SSI2Write(unsigned char data)

{

GPIOB->DATA &= ~0x20;       /* assert SS low */

SSI2->DR = data;            /* transmit high byte */

while(SSI2->SR & 0x10);     /* wait until transmit complete */

GPIOB->DATA |= 0x20;        /* keep SS idle high */

}

char SSI2Read()

{

char data;

while(SSI2->SR & 0x10);

data = SSI2->DR ;

return data;

}

void UART0Tx(char c)

{

while((UART0->FR & 0x0020) !=0){}

UART0->DR = c;

}

void sender(char arrr[])

{

int i=0;

for (i=0;i<strlen(arrr);i++)

{

UART0Tx(arrr[i]);

}

}

int main(void)

{

init_SSI2();

uartINnit();

initPortF();

while(1)

{

sender("Value Received is : ");

value=SSI2Read();

if(value=='R')

GPIOF->DATA=0x2;

else if(value=='G')

GPIOF->DATA=0x8;

else if(value=='B')

GPIOF->DATA=0x4;

UART0Tx(value);

sender("\n");

}

}

/* delay n milliseconds (16 MHz CPU clock) */

void delayMs(int n)

{

int i, j;

for(i = 0 ; i < n; i++)

for(j = 0; j < 3180; j++)

{} /* do nothing for 1 ms */

}

void SystemInit()

{

SCB->CPACR|=0XF00000;

}

### ISSUES

There were no issues.

### CONCLUSION

This lab session was conducted successfully. The SPI (serial peripheral interface) is a bus interface incorporated in many devices such as ADC, DAC, and EEPROM.

### APPLICATIONS

SSI allows microcontrollers to communicate synchronously with peripheral devices and other microcontrollers. Another name for this protocol is Serial Peripheral Interface or SPI. In this modern embedded systems era, many sensors such as ADCs, DACs, Temperature and pressure sensors etc, provide data over SPI communication. This way we can take off processor and microcontroller load of data acquisition and directly take data measurements from these sensors through SPI communication.

### POST LAB

void init_SSI2(void)

{

SYSCTL_RCGCSSI_R |= 4; /* enable clock to SSI2 */

SYSCTL_RCGCGPIO_R |= 2; /* enable clock to GPIOB for SSI2 */

while((SYSCTL_PRGPIO_R&0x0002) == 0){};// ready?

/* configure PORTB 4,5,7*/

GPIO_PORTB_AMSEL_R &= ~0xB0; /* disable analog for these pins */

GPIO_PORTB_DEN_R |= 0xB0; /* and make them digital */

GPIO_PORTB_DIR_R |= 0xB0; /* and make them digital */

GPIO_PORTB_AFSEL_R |= 0xB0; /* enable alternate function */

GPIO_PORTB_PCTL_R |= 0x20220000;/* assign pins to SSI2 */

GPIO_PORTB_DATA_R |= 0x20 ; /* keep SS idle high */

/* SPI Master, SPO = 0, SPH = 0, clock = 16 MHz, 8 bit data */

SSI2_CR1_R = 0; /* disable SSI and make it master */

SSI2_CR0_R = 0x00003307;; /* 9600 SSI clock, SPI mode, 8 bit data,

SCR=0x04 */

SSI2_CC_R = 0; /* use system clock */

SSI2_CPSR_R = 0x20; /* prescaler divided by 0x20 */

SSI2_CR1_R |= 2; /* enable SSI2 */

}

void SSI2Write(unsigned char data)

{

GPIO_PORTB_DATA_R &= ~0x20; /* assert SS low */

SSI2_DR_R = data; /* transmit high byte */

while(SSI2_SR_R & 0x10); /* wait until transmit complete */

GPIO_PORTB_DATA_R |= 0x20; /* keep SS idle high */

}

void init_SSI2(void)

{

SYSCTL_RCGCSSI_R |= 4; /* enable clock to SSI2 */

SYSCTL_RCGCGPIO_R |= 2; /* enable clock to GPIOB for SSI2 */

while((SYSCTL_PRGPIO_R&0x0002) == 0){};// ready?

/* configure PORTB 4,5,6*/

GPIO_PORTB_AMSEL_R &= ~0x70; /* disable analog for these pins

*/

GPIO_PORTB_DEN_R |= 0x70; /* and make them digital */

GPIO_PORTB_DIR_R |= 0x00; /* and make them digital */

GPIO_PORTB_AFSEL_R |= 0x70; /* enable alternate function */

GPIO_PORTB_PCTL_R |= 0x02220000;/* assign pins to SSI2 */

/* SPI Master, SPO = 0, SPH = 0, clock = 16 MHz, 8 bit data */

SSI2_CR1_R = 0x04; /* disable SSI and make it slave */

SSI2_CR0_R = 0x00003307; /* 9600 SSI clock, SPI mode, 8 bit data,

SCR=0x04 */

SSI2_CC_R = 0; /* use system clock */

SSI2_CPSR_R = 0x20; /* prescaler divided by 0x20 */

SSI2_CR1_R |= 2; /* enable SSI1 */

}

char SSI2Read()

{

char data;

while(SSI2_SR_R & 0x10);

data = SSI2_DR_R ;

return data;

}