# EMBEDDED SYSTEMS LAB

FAROOQ AHMAD 								EXPERIMENT # 9
17L-4407									LAB REPORT # 9
EL-7B

### ANALOG TO DIGITAL CONVERTER

### INTRODUCTION

In this lab session, we were introduced to analog to digital converter. An analog to digital converter (ADC) converts an analog signal into digital form. Analog-to-digital converters are among the most widely used devices for data acquisition. Digital computers use binary (discrete) values, but in the physical world everything is analog (continuous). Temperature, pressure (wind or liquid), humidity, and velocity are a few examples of physical quantities that we deal with every day. A physical quantity is converted to electrical (voltage, current) signals using a device called a transducer. Transducers used to generate electrical outputs are also referred to as sensors. Sensors for temperature, velocity, pressure, light, and many other natural physical quantities produce an output that is voltage (or current). Therefore, we need an analog-to-digital converter to translate the analog signals to digital numbers so that the microcontroller can read and process the numbers.

### OBJECTIVE

The course learning objective of this lab session was to Learn to configure ADC module of TM4C and o learn interfacing sensors with TM4C.

### PROCEDURE

Configuring GPIO for ADC input

In using ADC, we must also configure the GPIO pins to allow the connection of an analog signal through the input

pin. In this regard, it is the same as all other peripherals. The steps are as follow:

1. Enable the clock to GPIO pin by using RCGCGPIO.

2. Set the GPIOAFSEL (GPIO alternate function) bit for ADC input pin to 1.

3. Configure AINx signal to be used as analog input by clearing the bit in the GPIODEN (GPIO Digital enable)

register.

4. Disable the Analog isolation circuit for ADC input pins by writing a 1 to the GPIOAMSEL register.

ADC Channel Pin

AIN0 PE3

AIN1 PE2

AIN2 PE1

AIN3 PE0

AIN4 PD3

AIN5 PD2

AIN6 PD1

AIN7 PD0

AIN8 PE5

AIN9 PE4

AIN10 PB4

AIN11 PB5

Configuring ADC and reading ADC channel

After the GPIO configuration, we need to take the following steps to configure the ADC for Sample Sequencer 3

(SS3):

1. Enable the clock to ADC0 or ADC1 modules using RCGCADC register of System Registers. The SYSCTL-

>RCGCADC |= 3 will enable the clock to both ADC0 and ADC1 modules. For ADC0 module use SYSCTL-

>RCGCADC |= 1

2. Disable the Sample Sequencer using the ADCACTSS register before changing the configuration of the

sequencer. ADC0->ACTSS &= ~8 disables the SS3.

3. Choose the software trigger using the ADCEMUX register. Use ADC0->EMUX &= ~0xF000 for software trigger.

4. Select the ADC input channel using the ADCSSMUXn register. In the case of SS3 we use the ADCSSMUX3.

For example, The ADC0->SSMUX3 = 0 selects AIN0 channel on pin PE3.

5. Select the single-ended option, one-conversion per sample, and raw interrupt bit for end-of-conversion using

the ADCSSCTL3 register. Use ADC0->SSCTL3 |= 6 for single-ended, one-conversion, and raw interrupt.

6. Enable the Sample Sequencer SS3 using ADCACTSS register. ADC0->ACTSS |= 8 enables the SS3.

7. Use ADCPSSI register to start a new conversion. Use ADC0->PSSI |= 8 to start a conversion by Sample

Sequencer 3.

8. Keep monitoring the end-of-conversion INT3 flag in ADCRIS register.

9. When the INT3 goes HIGH, read the ADC result from the ADCSSFIFO3 and save it.

10. After reading the ADC result in step 9, clear the INT3 flag in ADCRIS register to allow for the next conversion.

To clear INT3 flag, write to ADCICS register with ADC0->ISC = 8.

11. Repeat steps 8 through 10 for the next conversion.

### ISSUES

There were no issues.

### CONCLUSION

This experiment was performed successfully. In this lab session, it was learnt how to configure the analog to digital (ADC) module of tm4c123gh6pm tiva launchpad and with the use of it how to perform interfacing of the various sensors with tm4c. Analog-to-digital conversion (ADC) is necessary because, while embedded systems deal with digital values, their surroundings typically involve many analog signals such as, temperature, speed, pressure, the output of a microphone, etc. They all need to be converted into digital data before being processed by the microcontroller.

### APPLICATIONS

An analog to digital converter (ADC) is an electronic device which converts varying analog signals into digital signals so that they can easily be read by the digital devices. It has many applications in electronics projects. ADC converts the quantities of real-world phenomenon in to digital language which is used in control systems, data computing, data transmission and information processing. Figure below shows the input/ output relationship of an ADC. Analog-to-digital conversion (ADC) is necessary because, while embedded systems deal with digital values, their surroundings typically involve many analog signals such as, temperature, speed, pressure, the output of a microphone, etc. They all need to be converted into digital data before being processed by the microcontroller. An embedded system uses the ADC to collect information about the external world (data acquisition system.) The input signal is usually an analog voltage, and the output is a binary number.

### POST LAB

#include <stdint.h>

#include "inc/tm4c123gh6pm.h"

void ADC_Init(void);

void DisableInterrupts(void);

void EnableInterrupts(void);

void WaitForInterrupt(void);

void GPIOPortF_Init(void);

void GPIOPortF_Handler(void);

void delayMs(int);

void ADC_Init(void)

{

SYSCTL_RCGCGPIO_R |= 0x8; //Unlocking Port D

SYSCTL_RCGCADC_R |=3; //Enabling ADC Module 0 & 1

GPIO_PORTD_LOCK_R = 0x4C4F434B; //Unlock

GPIO_PORTD_AFSEL_R |=3; //Alternate function for PD1-0

GPIO_PORTD_DEN_R &=~3; //Digital Disable PD1-0

GPIO_PORTD_AMSEL_R |=3; //Analog Enable PD1-0

ADC0_ACTSS_R &=~8; //Disable ADC0 seq3

ADC0_EMUX_R&=~0xF000; //Use default processor

ADC0_SSMUX3_R=6; // ADC input 6 at PD1

ADC0_SSCTL3_R |=6; //take one sample at a time, set flag at 1st sample

ADC0_ACTSS_R |=8; //Enable ADC0 seq3

NVIC_EN0_R |= (1<<17); //IRQ of ADC0 SS3

ADC0_IM_R |=8; //sample Sequencer 3

ADC1_ACTSS_R &=~8; //Disable ADC1 seq3

ADC1_EMUX_R&=~0xF000; //Use default processor

ADC1_SSMUX3_R=7; //ADC input 7 at PD0

ADC1_SSCTL3_R |=6; //take one sample at a time, set flag at 1st sample ADC1_ACTSS_R |=8; //Enable ADC1 seq3

NVIC_EN1_R |= (1<<19); //IRQ of ADC1 SS3

ADC1_IM_R |=8; //sample Sequencer 3

}

void GPIOPortF_Init(void)

{

SYSCTL_RCGC2_R |= 0x00000020;   /* 1) activate clock for PortF */

GPIO_PORTF_LOCK_R = 0x4C4F434B; /* 2) unlock GPIO PortF */

GPIO_PORTF_CR_R = 0x1E;         /* allow changes to PF4-0 */

GPIO_PORTF_AMSEL_R = 0x00;      /* 3) disable analog on PF */

GPIO_PORTF_PCTL_R = 0x00000000; /* 4) PCTL GPIO on PF4-0 */

GPIO_PORTF_DIR_R &= 0xEF;         /* PF4 output  1110_1111*/

GPIO_PORTF_DIR_R |= 0x0E;         /* make PORTF3, 2, 1 output for LEDs */

GPIO_PORTF_AFSEL_R = 0x00;      /* 6) disable alt funct on PF7-0 */

GPIO_PORTF_PUR_R = 0x10;        /* enable pull-up on PF0 and PF4 */

GPIO_PORTF_DEN_R = 0x1E;        /* 7) enable digital I/O on PF4-0 */

//interrupt related settings

GPIO_PORTF_IS_R &= ~0x10;       /*  PF4 is edge-sensitive */

GPIO_PORTF_IEV_R &= ~0x10;      /*  PF4 falling edge event */

GPIO_PORTF_ICR_R = 0x10;        /*  Clear flag4 */

GPIO_PORTF_IM_R |= 0x10;        /*  arm interrupt on PF4 */

NVIC_EN0_R = 0x40000000;        /*  Enable interrupt 30 in NVIC */

EnableInterrupts();             /* Enable global Interrupt flag (I) */

}

/* delay n milliseconds (16 MHz CPU clock) */

void delayMs(int n)

{

int i, j;

for(i = 0 ; i < n; i++)

for(j = 0; j < 3180; j++)

{}  /* do nothing for 1 ms */

}

/*********** DisableInterrupts ***************

*

* disable interrupts

*

* inputs:  none

* outputs: none

*/

void DisableInterrupts(void)

{

__asm(" cpsid   i\n"          );

}

/*********** EnableInterrupts ***************

*

* enable interrupts

*

* inputs:  none

* outputs: none

*/

void EnableInterrupts(void)

{

__asm( " cpsie   i\n");

}